PRE LAB 2

Exercise 1:

library ieee;

use ieee.std_logic_1164.all;

entity PreEx1 is

port(a, b, ci: in  std_logic;

co, s: out std_logic);

end PreEx1;

architecture Full_Adder of PreEx1 is

begin

s <= a xor (b xor ci);

co <= (a and (b or ci)) or (b and ci);

end Full_Adder;

Exercise 2:

library ieee;

use ieee.std_logic_1164.all;

entity PreEx2 is

port(    R,  S, CLK: in     std_logic;

Rg, Sg, Qa, Qb: buffer std_logic);

end PreEx2;

architecture RS_Latch of PreEx2 is

begin

Rg <= R and CLK;

Sg <= S and CLK;

Qa <= Rg nor Qb;

Qb <= Sg nor Qa;

end RS_Latch;

Exercise 3:

library ieee;

use ieee.std_logic_1164.all;

entity PreEx3 is

port(        D, CLK: in     std_logic;

Rg, Sg, Qa, Qb: buffer std_logic);

end PreEx3;

architecture D_Latch of PreEx3 is

begin

Rg <= (not D) nand CLK;

Sg <= D nand CLK;

Qa <= Sg nand Qb;

Qb <= Rg nand Qa;

end D_Latch;